MeltR is a Software Package that Provides Facile Determination of Biopolymer Thermodynamics: Application to RNA UV-Absorbance Melting Data

Jacob P. Sieg1,2, Sebastian J. Arteaga3, Brent M. Znosko3, Philip C. Bevilacqua*1,2,4

1Department of Chemistry, Pennsylvania State University, University Park, PA 16802.

2Center for RNA Molecular Biology, Pennsylvania State University, University Park, PA 16802.

3Department of Chemistry, Saint Louis University, Saint Louis, MO 63103.

4Department of Biochemistry and Molecular Biology, Pennsylvania State University, University Park, PA 16802.

*Correspondence should be directed to pcb5@psu.edu

Keywords: Absorbance melting curves, thermodynamics, RNA

# Abstract

Themodenaturation (melting) curves of macromolecules are used to determine folding thermodynamic parameters. Notably, this thermodynamic insight into RNA and DNA structure underlies a plethora of structure prediction tools. Appropriate analysis of absorbance melting curves is complex, requiring many data preprocessing, regression, and error analysis steps. The absorbance melting curve-fitting software MeltWin, introduced in 1996, provided researchers with a consistent and facile absorbance melting curve analysis platform that used in a generation of RNA folding parameters. Unfortunately, MeltWin software has non-transparent source code, is not maintained, and relies on idiosyncratic choices by the user. Herein, we provide MeltR, an open-source, curve-fitting package in the popular R programming language for analysis of macromolecular thermodynamics data. The MeltR provides the same facile conversion of absorbance melting curve data to folding energies provided by MeltWin but has additional features for experiments. MeltR can be a useful tool for analyzing the next generation of DNA and RNA absorbance melting curve experiments, and can be used to analyze other types of melting data, collected on diverse biological macromolecules.

# Why it matters

Melting curves provide the thermodynamic parameters used in a plethora of algorithms that predict nucleic acid structure and behavior. Appropriate analysis of melting curves is complex and hard to perform on a large scale. Herein, we provide MeltR, a replacement and improvement on the popular melting-curve fitting software, MeltWin. MeltR uses a facile command-line interface, automates idiosyncrasies in data analysis, and a MeltR analysis is easy to distribute and reproduce. Lastly, MeltR is written in the popular R statistical programming language and can be copied, improved, and redistributed without restriction. We believe that MeltR is a valuable tool for analyzing melting data collected on diverse biological macromolecules, particularly RNA and DNA.

# Introduction

Thermodenaturation (melting) curves monitor macromolecular structure as a function of temperature and provide valuable thermodynamic insight. UV-detected absorbance melting curves of nucleic acid helices have provided extensive thermodynamic characterization of the sequence dependence RNA and DNA structure.1,2 This thermodynamic characterization has provided the nearest neighbor model for calculating the energy of nucleic acid structure,3–8 which underlies a plethora of popular algorithms for predicting the behavior of nucleic acids (Figure 1A).9–15

Extraction of thermodynamic parameters from raw melting data requires many preprocessing, regression, and error analysis steps, which can be difficult to achieve manually on a large scale (Figure 1B).16 The program MeltWin,17 introduced in 1996, provided researchers with a consistent and facile data analysis platform, which underlies the aforementioned thermodynamic characterization of nucleic acid structure. While the current characterization of nucleic acid structure in many environments is extensive, in particular RNA secondary structure in 1 M NaCl,8 a lot of questions remain. In particular, the thermodynamic contribution of the cellular environment18,19 and naturally-modified nucleotides20 are areas of active research. Unfortunately, Meltwin is no-longer maintained, has non-transparent algorithms, and relies on idiosyncratic choices by the user to process data.

Herein, we introduce MeltR: an open-source package in the popular R programming language (https://www.R-project.org/) that provides facile conversion of raw data to thermodynamic parameters. MeltR provides fitting protocols for fluorescence binding isotherms (previously reported)21 and melting curves similar to MeltWin. Herein, we describe the application of MeltR for fitting UV-detected absorbance melting curves of RNA, describe unique features to automate data processing not provided by MeltWin, and benchmark the accuracy of MeltR on real and modeled data.

Using MeltR requires a little knowledge of the R programming language. This knowledge is easily picked up from the help provided at https://github.com/JPSieg/MeltR. A small number of R specific definitions that are helpful for the reader of this paper are underlined and provided at the end of the text.

# Results

## Facile fitting of raw absorbance data with meltR.A

MeltR includes the function meltR.A, which provides a comparable analysis to MeltWin. The meltR.A function requires 3 user-defined arguments: data set, nucleic acid, and molecular model (Figure 1C). The first argument provides the data set, which is produced by reading a tidy formatted, text file into a R data frame (Figure 1C.1). The second essential argument specifies the nucleic acid in the melt. This is provided in R as a vector and is used to determine the extinction coefficient of the nucleic acid (Figure 1C.2). The first element of the vector is the type of nucleic acid, either “DNA” or “RNA”, and the remaining elements are the individual residues in the sample. Alternatively, the first element can be “custom”, and the remaining elements can be custom extinction coefficients in M-1cm-1 or can be a series of concentrations for each sample for non-absorbance data The third element is the molecular model, either "Monomolecular.2State", "Heteroduplex.2State", or "Homoduplex.2State", for a single self-structured strand, two non-self-complementary strands forming a duplex, or a bimolecular association of a self-complementary strand forming a duplex, respectively (Figure 1C.3). The meltR.A function has other arguments with default settings that can be adjusted. Importantly, the “concT” is the temperature used to calculate the concentration (Figure 1C.4) and “fitTs” is the temperature range used in the fit (Figure 1C.5). The “fitTs” argument is used for baseline trimming (discussed in detail below), and can be a vector containing the temperature range to be fit for all samples. Alternatively, the “fitTs” argument can be a list of vectors specifying a temperature range for each sample.

The meltR.A function provides three analyses: individual fits, Tm vs. concentration (Ct), and global fitting (Figure 1D). The first analysis is individual fits (method 1), which fits each sample to an individual model to calculate thermodynamic parameters (Figure 1D.1). For method 1, the averages of thermodynamic energies are reported (Figure 1D.2). The second analysis, Tm vs. Ct (method 2), fits the relationship between the Tm and the Ct from different samples to calculate thermodynamic parameters (Figure 1D.2). Methods 1 and 2 were both provided by MeltWin.17 The meltR.A function also provides a third method, global-fitting analysis, not provided by MeltWin (Figure 1B). Global fitting combines methods 1 and 2 by directly fitting the raw absorbance melting curves, similar to method 1, but uses information from every sample in a single model, similar to method 2. Figure 1C shows the response generated when a user fits a data set. Note, the response includes the %error between methods (Figure 1D.3), which is the range between methods 1, 2, and 3, divided by the average. This metric is useful for assessing the overall quality of fits, and two-state folding where a %error between methods greater than 10% is diagnostic of a multistate-folding transition. Lastly, meltR.A saves the result of a fit to figures and tables to help the user assess and present results (Figure S1).

Several other meltR.A features are described in the help file (File S2). Notably, meltR.A can use an alternative method to calculate the Tm for each sample, calculate extinction coefficients at different wavelengths, and turn off methods 2 and 3. Turning off methods 2 and 3 is useful for experiments that compare absorbance melting curves between conditions and RNA. Method 2 is turned off automatically for monomolecular systems because there is no dependence between the Tm and concentration. Likewise, meltR.A generates an extensive fit object in R, consisting of a list of data and statistics (File S2). The meltR.A fit object is included for advanced analysis of the data, including regression statistics, residuals, and useful transformations of the data such as first and second derivatives. Elements of the meltR.A fit object can be passed to other analyses.

## MeltR provides a consistent, rational, and statistically appropriate baseline-trimming protocol

Both MeltWin and MeltR approximate the absorbance of the folded and unfolded states as lines.16 These lines approximate absorbance changes due to changes in base stacking in the duplex and single stranded states with temperature. Baselines should be trimmed to exclude aberrant data and to include no more data than is required to define the duplex and single stranded state, thus avoiding extrapolation of baselines from temperatures that are far removed from the melt region. However, users must make idiosyncratic choices about how to trim manually, as no protocols have been established.22

One approach is to trim to a constant widow surrounding the melt region, for example 20 °C. Alternatively, a user could identify an optimum baseline range by iteratively fitting the data using different baseline ranges with MeltWin or meltR.A, and then choose the baseline range that provides the best internal consistency across the data set by assessing agreement between methods. Unfortunately, both approaches are idiosyncratic. We thus sought to provide an auto-baseline trimmer in MeltR that gives users a consistent and rational baseline-trimming protocol.

MeltR provides a baseline trimming function, the “BLtrimmer”, that works on three principles:

1. A large number of combinations of trimmed-baselines are generated and fit.

2. The best baseline combinations produce the most internally consistent folding energy estimates.

3. Thermodynamic energies from the best combinations should be treated as an ensemble of equally feasible estimates.

The BLtrimmer has only one required argument, a MeltR fit object produced by meltR.A (Figure 2A.) and prints an analysis of thermodynamic parameters produced by an ensemble of baseline combinations (Figure 2B). The BLTrimmer also provides an analysis of all the tested baselines (Figure S2).

The other BLTrimmer arguments are adjustable but set to the recommended value by default (Figure 2A.2-8). The first adjustable argument is the “Trim.method”, which corresponds to the method used to generate baselines, either “floating” to allow different baseline lengths for each sample or “fixed” to use the same baseline length for each sample (Figure 2DA.2). The “Assess.method” argument corresponds to the method used to evaluate baseline combinations for internal consistency (Figure 2DA.3). It can be 1 of 3 integer valu es, “1” to optimize agreement between samples in method 1, “2” to optimize agreement between methods 1 and 2, or “3” to optimize both agreement between method 1 and agreement between methods 1 and 2. The “no.trim.range” argument defines the core melt region, by default between 10% and 90% unfolded (Figure 2A.4). The “quantile.threshold” argument, corresponds to the fraction of baseline combinations to pass to the final ensemble (Figure 2A.5). By default, the BLtrimmer will use the 25% of baseline combinations that produce the most consistent parameters.

The final adjustable arguments determine how baselines are generated and either apply to the fixed or floating trim method. The “n.ranges” argument is the number of ranges to generate per sample and the “range.step” argument is the temperature step between baselines. By default, using the floating method, the BLtrimmer will produce 5 symmetrical baselines per sample, at lengths of 5, 10, 15, 20, and 25 °C. The “n.combinations” argument allows the user to specify the number of random baseline combinations to test by default 1000. Testing 1000 random “floating” combinations is enough to achieve convergence in about 1 min (Figure S3).

## MeltR accurately reproduces thermodynamic parameters

We first determined that MeltR accurately determined folding energies by fitting modeled data. Nine realistic (see File S1 methods) absorbance melting curve were modeled for 96 two-state folding helices.7,23 The modeled data were then fit with meltR.A followed by the BLtrimmer with no manual baseline trimming. BLtrimmer results for all three methods were in good agreement with the known folding energies (Figure S4A-D).

We next tested MeltR by fitting real data. We compiled five previously published data sets of data for self-complementary RNA,19 and six new data sets collected on a non-self-complementary RNA, which are presented here. The data were fit with meltR.A and these fits were passed through BLtrimmer, (Table S1). The three methods were in good agreement, with an average ΔH°, ΔS°, ΔG°37 and Tm error between methods of 4.8%, 5.4% , 1.2%, and 0.4%, respectively.

To obtain an independent measure of accuracy, we compared the parameters we calculated with MeltR to those calculated with MeltWin. Parameters calculated with methods 1 and 2 using MeltWin were obtained from the published source19 for the self-complementary duplexes and are presented here for heteroduplexes (Table S1). MeltR method 1 was in good agreement with MeltWin method 1 with an average ΔH°, ΔS°, ΔG°37, and Tm error between programs of 1.8%, 2.0%, 2.5%, and 2.2%, respectively (Figure 3A). Likewise, MeltR method 2 was in good agreement with MeltWin method 2 with an average ΔH°, ΔS°, ΔG°37 and Tm error between programs of 3.0%, 3.3%, 2.6%, and 2.2%, respectively (Figure 3A). Interestingly, MeltR method 3 reasonably reconstituted both MeltWin methods 1 and 2, even though it is an original method (Figure 3B-F).

We next tested MeltR for agreement with the consensus nearest neighbor model. Folding energies were calculated using published Watson-Crick nearest neighbor parameters for each helix.7,19 Errors were within 10% of the ΔG°37 provided by the nearest neighbor model and there was no significant difference between the distribution of errors between any of the methods for MeltR and MeltWin (Figure S4E).

## The MeltR provides a two-state folding test

Helices are tested for two-state folding by comparing the ΔH° agreement between methods 1 and 2. If the difference is smaller than 10% of the mean value, the helix is considered two-state. However, the results of methods 1 and 2 are both sensitive to baseline-trimming (Figure S2E). This baseline sensitivity leads to a problem, determining if the helix is truly two-state folding or if errors in baseline trimming caused the appearance of two-state folding. The BLTrimmer provides a solution to this problem, based on analysis of the distribution of helix energies derived from randomly trimmed baselines.

For example, we compared the distribution of ΔH° agreement for a helix that is expected to exhibit strong two-state folding (Figure 5A) and a helix is expected to exhibit non-two-state folding (Figure 4B). This non-two-state folding helix consisted of a GC rich helix and an AU rich helix, separated by a 2 nucleotide internal loop. Thus, the AU rich end is expected to melt independently from the GC rich end, thus populating a third state during the melting transition. We then examined the distribution of the ΔH° agreement between method 1 and method 2 for 1000 random baseline combinations (Figure 5C). The distribution peaks below 10% for the two-state folding sequence and the distribution beaks above 10% for the non-two-state folding sequence. This distribution analysis provides a less biased metric of two-state folding than analysis of a single baseline combination.

# Discussion

We determined that MeltR was accurate in comparison to MeltWin, a similar program. An uncertainty of 6%, 6%, and 3% for the ΔH°, ΔS°, and ΔG°37 is expected for UV-detected absorbance melting curve analysis of nucleic acids, from systematic errors such as faulty instrument calibration and discrepancies in data analyses.7,16 We found that MeltR was able to reconstitute analysis with minimal user intervention with errors smaller than 4.1%, 4.6%, and 3% for the ΔH°, ΔS°, and ΔG°37, respectively, even using independent methods to calculate thermodynamic parameters (Figure 3A). Errors between programs using the same method were even smaller, and likely reflect discrepancies in baseline trimming choices between manual trimming in MeltWin and automated baseline trimming in MeltR.

Baseline trimming is an important but poorly understood aspect of extracting thermodynamic parameters from melting curves.22 Using default parameters, the BLTrimmer produces  accurate thermodynamic parameters with minimal user specifications (Figure 3). Moreover, the default parameters can be adjusted by users interested in establishing a unique baseline-trimming protocol. Thus, MeltR provides a consistent, rational, and statistically appropriate baseline-trimming method.

Interestingly, we determined that method 2, fitting the relationship between Tm and lnCt, was less accurate for fitting modeled data (Figure S4A-D). Less accurate fitting of modeled data by method 2 indicates that method 2 is more sensitive to random fluctuations in absorbance measurements, because that is the only type of error incorporated into the model. In general, method 2 is favored by the nucleic acid thermodynamics field, because it links all melting curves in the data set to a single regression model. Our results indicate that this preference should be reconsidered.

The BLTrimmer identifies an ensemble of optimum baseline combinations via agreement between different methods. An obvious concern is that baselines will be selected in a way that causes a non-two-state data set to appear two-state. In our experience, this false selection does not occur using the BLTrimmer with default parameters, as the ΔH° distribution from methods 1 and 2 do not overlap for a true non-two-state system (Figure 4D). However, users should check the distribution of ΔH° agreement from a large number of baseline combinations for non-two-state behavior. We also recommend testing all non-default BLTrimmer settings with a true non-two-state data set, as a positive control provided here (https://github.com/JPSieg/meltR.a.paper/blob/main/Figures/Figure_4_BLTrimmer_two_state_check/js6040_multistate_data.csv).

MeltR is written in the popular R programming language, is open source, and provides the following advantages. First, MeltR is as accurate as the comparable software MeltWin, even though MeltR requires less user intervention. Second, MeltR is inherently transparent because the source code is written in an accessible programming language. The source code can be modified and redistributed without restrictions, providing a platform for others to improve. Second, MeltR provides transparent analysis because it can be installed on any computer with R. The MeltR commands used to analyze a data set can be saved and distributed in a script which another researcher can then open and reproduce. Lastly, MeltR could provide a platform for facile and reproducible analysis of diverse thermal denaturation experiments. Thus, MeltR is a useful tool for analyzing melting curve experiments.

# Materials and Methods

## Data transparency and reproducibility

Data were analyzed in R (V4.2.1). All raw data files, analysis code, and figures are available at https://github.com/JPSieg/meltR.a.paper. MeltR is availible at https://github.com/JPSieg/MeltR and and V1.0 is archived at “insert Zenodo URL”.

## Absorbance melting curves

For published data, unmodified data files from the instrument were compiled from the original publication and parsed into a tidy format. For original experiments, RNA was ordered from Integrated DNA Technology with RNase free HPLC purification, resuspended, and buffer exchanged into 1 M NaCl, 20 mM MOPS pH 7.0, 0.01 mM EDTA, 0.001% (w/v) SDS buffer. Samples were diluted in buffer to their final volume and melting experiments were performed on an OILS-refurbished HP 8425 diode array spectrophotometer from 5 to 95 °C at a ramp rate of 0.5 °C/min. Modeled absorbance melting curves were simulated to have realistic random scatter, random baseline slopes, and random intercepts, as described in the File S1 methods. Data were fit with MeltWin as described previously,19 and with MeltR using the minimum number of user specified parameters. A detailed description of the methods used by meltR.A and the BLTrimmer is provided in the S File S1 methods.

# Supplemental Materials

File S1: supplemental methods, supplemental figures, and supplemental tables.

File S2: meltR.A help file

# Definitions

Function: Code that automates a task. Argument: The input a function needs to run. Tidy formatted: A consistent data format where each variable has its own column. Each observation has its own row. Each value has its own cell. Data frame: A table of data in R. Specifically, a list of vectors of the same length, each containing a different variable. Vector: Basic unit of data storage in R. A vector is created with the “c” function. For example, to create a vector for the “NucAcid” argument for meltR.A, one can use the “c” function, “c(“RNA”, “AAAA”, “UUUU”)”. List: A more advanced type of data storage in R and can be a list of anything, including vectors, data frames, and other lists.

# Acknowledgments

This work was supported by a National Institutes of Health Grant R35-GM127064 to PCB. We thank Dr. Susan Schroeder for the discussion and feedback concerning MeltR.

# Author contributions

All authors contributed to the preparation of this manuscript.

# Declaration of interests

All authors have no competing interests to declare.

# References

(1)	Puglisi, J. D.; Tinoco, I. Absorbance Melting Curves of RNA. In Methods in Enzymology; RNA Processing Part A: General Methods; Academic Press, 1989; Vol. 180, pp 304–325. https://doi.org/10.1016/0076-6879(89)80108-9.

(2)	Schroeder, S. J.; Turner, D. H. Optical Melting Measurements of Nucleic Acid Thermodynamics. Methods Enzymol. 2009, 468, 371–387. https://doi.org/10.1016/S0076-6879(09)68017-4.

(3)	Tinoco, I.; Uhlenbeck, O. C.; Levine, M. D. Estimation of Secondary Structure in Ribonucleic Acids. Nature 1971, 230 (5293), 362–367. https://doi.org/10.1038/230362a0.

(4)	Zuker, M.; Sankoff, D. RNA Secondary Structures and Their Prediction. Bull. Math. Biol. 1984, 46 (4), 591–621.

(5)	Sugimoto, N.; Nakano, S.; Katoh, M.; Matsumura, A.; Nakamuta, H.; Ohmichi, T.; Yoneyama, M.; Sasaki, M. Thermodynamic Parameters To Predict Stability of RNA/DNA Hybrid Duplexes. Biochemistry 1995, 34 (35), 11211–11216. https://doi.org/10.1021/bi00035a029.

(6)	SantaLucia, John; Allawi, H. T.; Seneviratne, P. A. Improved Nearest-Neighbor Parameters for Predicting DNA Duplex Stability. Biochemistry 1996, 35 (11), 3555–3562. https://doi.org/10.1021/bi951907q.

(7)	Xia, T.; SantaLucia, J.; Burkard, M. E.; Kierzek, R.; Schroeder, S. J.; Jiao, X.; Cox, C.; Turner, D. H. Thermodynamic Parameters for an Expanded Nearest-Neighbor Model for Formation of RNA Duplexes with Watson−Crick Base Pairs †. Biochemistry 1998, 37 (42), 14719–14735. https://doi.org/10.1021/bi9809425.

(8)	Turner, D. H.; Mathews, D. H. NNDB: The Nearest Neighbor Parameter Database for Predicting Stability of Nucleic Acid Secondary Structure. Nucleic Acids Res. 2010, 38 (Database issue), D280–D282. https://doi.org/10.1093/nar/gkp892.

(9)	Zuker, M. Mfold Web Server for Nucleic Acid Folding and Hybridization Prediction. Nucleic Acids Res. 2003, 31 (13), 3406–3415.

(10)	Markham, N. R.; Zuker, M. DINAMelt Web Server for Nucleic Acid Melting Prediction. Nucleic Acids Res. 2005, 33 (suppl_2), W577–W581. https://doi.org/10.1093/nar/gki591.

(11)	Gruber, A. R.; Lorenz, R.; Bernhart, S. H.; Neuböck, R.; Hofacker, I. L. The Vienna RNA Websuite. Nucleic Acids Res. 2008, 36 (Web Server issue), W70–W74. https://doi.org/10.1093/nar/gkn188.

(12)	Reuter, J. S.; Mathews, D. H. RNAstructure: Software for RNA Secondary Structure Prediction and Analysis. BMC Bioinformatics 2010, 11, 129. https://doi.org/10.1186/1471-2105-11-129.

(13)	Zadeh, J. N.; Steenberg, C. D.; Bois, J. S.; Wolfe, B. R.; Pierce, M. B.; Khan, A. R.; Dirks, R. M.; Pierce, N. A. NUPACK: Analysis and Design of Nucleic Acid Systems. J. Comput. Chem. 2011, 32 (1), 170–173. https://doi.org/10.1002/jcc.21596.

(14)	Ye, J.; Coulouris, G.; Zaretskaya, I.; Cutcutache, I.; Rozen, S.; Madden, T. L. Primer-BLAST: A Tool to Design Target-Specific Primers for Polymerase Chain Reaction. BMC Bioinformatics 2012, 13 (1), 134. https://doi.org/10.1186/1471-2105-13-134.

(15)	Peng, D.; Tarleton, R. 2015. EuPaGDT: A Web Tool Tailored to Design CRISPR Guide RNAs for Eukaryotic Pathogens. Microb. Genomics 1 (4), e000033. https://doi.org/10.1099/mgen.0.000033.

(16)	SantaLucia Jr., J.; Turner, D. H. Measuring the Thermodynamics of RNA Secondary Structure Formation. Biopolymers 1997, 44 (3), 309–319. https://doi.org/10.1002/(SICI)1097-0282(1997)44:3<309::AID-BIP8>3.0.CO;2-Z.

(17)	McDowell, J. A.; Turner, D. H. Investigation of the Structural Basis for Thermodynamic Stabilities of Tandem GU Mismatches: Solution Structure of (RGAG GU CUC) 2 by Two-Dimensional NMR and Simulated Annealing ,. Biochemistry 1996, 35 (45), 14077–14089. https://doi.org/10.1021/bi9615710.

(18)	Ghosh, S.; Takahashi, S.; Ohyama, T.; Endoh, T.; Tateishi-Karimata, H.; Sugimoto, N. Nearest-Neighbor Parameters for Predicting DNA Duplex Stability in Diverse Molecular Crowding Conditions. Proc. Natl. Acad. Sci. 2020, 117 (25), 14194–14201. https://doi.org/10.1073/pnas.1920886117.

(19)	Adams, M. S.; Znosko, B. M. Thermodynamic Characterization and Nearest Neighbor Parameters for RNA Duplexes under Molecular Crowding Conditions. Nucleic Acids Res. 2019, 47 (7), 3658–3666. https://doi.org/10.1093/nar/gkz019.

(20)	Hopfinger, M. C.; Kirkpatrick, C. C.; Znosko, B. M. Predictions and Analyses of RNA Nearest Neighbor Parameters for Modified Nucleotides. Nucleic Acids Res. 2020, 48 (16), 8901–8913. https://doi.org/10.1093/nar/gkaa654.

(21)	Sieg, J. P.; McKinley, L. N.; Huot, M. J.; Yennawar, N. H.; Bevilacqua, P. C. The Metabolome Weakens RNA Thermodynamic Stability and Strengthens RNA Chemical Stability. Biochemistry 2022. https://doi.org/10.1021/acs.biochem.2c00488.

(22)	Majikes, J. M.; Zwolak, M.; Liddle, J. A. Best Practice for Improved Accuracy: A Critical Reassessment of van’t Hoff Analysis of Melt Curves. Biophys. J. 2022, 121 (11), 1986–2001. https://doi.org/10.1016/j.bpj.2022.05.008.

(23)	Antao, V. P.; Tinoco, I., Jr. Thermodynamic Parameters for Loop Formation in RNA and DNA Hairpin Tetraloops. Nucleic Acids Res. 1992, 20 (4), 819–824. https://doi.org/10.1093/nar/20.4.819.

Figure 1  Determination of thermodynamic parameters from absorbance melting data using meltR.A in the MeltR package. (A) Themodenaturation data underlies algorithms that predict nucleic acid structure and behavior. (B) Raw data to thermodynamic parameter conversion is automated with MeltR. (C) The usage of meltR.A in an R script. (1) The data frame (df) containing the absorbance data (2) A vector specifying the nucleic acid. (3) Duplex type, either "Monomolecular.2State", "Heteroduplex.2State”, or "Homoduplex.2State”, (4) The temperature used to calculate the RNA concentration. (5) The temperature range that is fit, used for manual baseline trimming. (D) The raw output of meltR.A.

Figure 2  Auto-baseline trimming absorbance data using the BLtrimmer. (A) BLtrimmer usage in a R script. (1) A MeltR fit object from meltR.A. (2) Method used to generate random baselines. Either “fixed” or “floating”. (3) The method used to asses each baseline range for internal consistency across samples. (4) Defines the core of the melt region which is not trimmed. (5) The fraction of optimum baseline combinations to analyze in the final ensemble. (6) The number of baseline ranges the BLtrimmer will produce for each sample. (7) The temperature difference for each baseline produced on an absorbance melting curve. (8) The number of baseline combinations to test in a given BLtrimmer run. (B) The raw output of the BLtrimmer.

Figure 3  MeltR functions meltR.A and BLTrimmer reproduce folding energies calculated using MeltWin. (A) Average error between MeltR and MeltWin for each method. (B-F) Thermodynamic parameters determined from method 1 (purple) and method 2 (green) using MeltWin versus parameters determined using method 3, global fitting,in MeltR. Horizontal error bars represent precision in the parameters calculated by MeltWin and vertical error bars represent 95% confidence intervals calculated using the BLtrimmer.

Figure 4  Two-state folding test using the BLtrimmer in MeltR. (A) Melting transition for a two-state folding sequence. (B) Melting transition for a non-two-state folding sequence. (C) The distribution of the agreement between the ΔH° using methods 1 (average of fitting samples individually) and method 2 (1/Tm versus lnCt).