Supplementary Information

MeltR is a Software Package that Provides Facile Determination of Biopolymer Thermodynamics: Application to RNA UV-Absorbance Data

Jacob P. Sieg1,2, Sebastian J. Arteaga3, Brent M. Znosko3, Philip C. Bevilacqua1,2,4

1Department of Chemistry, Pennsylvania State University, University Park, PA 16802.

2Center for RNA Molecular Biology, Pennsylvania State University, University Park, PA 16802.

3Department of Chemistry, Saint Louis University, Saint Louis, MO 63103.

4Department of Biochemistry and Molecular Biology, Pennsylvania State University, University Park, PA 16802.

Keywords: Absorbance melting curves, thermodynamics, RNA

ORCID:

Jacob P. Sieg: 0000-0001-5414-1667

Sebastian J. Arteaga:

Brent M. Znosko:

Philip C. Bevilacqua: 0000-0001-8074-3434

*Corresponding author

Table of Contents

Supplementary information methods	3

Data processing by meltR.A	3

Raw data preprocessing	3

Thermodynamic models for duplex formation.	3

Method 1: fitting melting curves individually.	4

Method 2: fitting the Tm as a function of Ct.	5

Method 3: Global fitting.	5

Data processing by the BLtrimmer	5

Baseline testing.	5

Baseline assessment.	6

Ensemble analysis	6

Data Modeling	6

Supplementary information figures	8

Figure S1	8

Figure S2	9

Figure S3	10

Figure S4	11

Table S1	12

Supplementary information references	15

# Supplementary information methods

## Data processing by meltR.A

Raw data preprocessing. MeltR performs the following data preprocessing steps before fitting:

1.) RNA and DNA extinction coefficients are calculated from the sequence using the extinction coefficient data1 and the method recommended by Tinoco and colleagues.2 Alternatively, the user provides custom extinction coefficients or concentrations for each non-blank sample in the data set.

2.) If the blanks are provided in the data set, the background absorbance of a user specified blank is scaled to the pathlength of the cuvette and subtracted from each curve.

3.) If concentrations were not supplied in step 1, the total strand concentration (Ct) is calculated at a user specified temperature (Default = 90 °C) using the extinction coefficient and Beer’s law. For homoduplexes and monomolecular-self structured RNA, where there is only one nucleic acid in the sample, the Ct is the total concentration of the single strand. For heteroduplexes, the Ct is the total concentration of strand 1 plus the total concentration of strand 2.

4.) High and low temperature data are trimmed (to the user’s specification) to ensure linear baselines.

5.) First and second derivatives are taken using polynomial regression. First, the data are approximated using a 20th order polynomial. Then, the first and second derivatives of the polynomial are determined analytically using calculus. The approximate T0.5 (the approximate melting temperature, Tm, where 50% of the nucleic acid is single stranded) is calculated by finding the maximum of the first derivative curve and the T0.75 (the approximate temperature where 75% of the nucleic acid is single stranded) is calculated by finding the minimum of the second derivative (Figure 2), to a precision of less than 0.1 °C.

6.) Initial parameter estimates are calculated for each curve. The initial values for slopes and intercepts of the baselines are estimated by fitting absorbance values that are greater than the 75th quantile for the uper baselines, and fitting aborbance values that are lower than the 25th quantile for the lower baseline, to y = mx + b. Initial values for the enthalpy are determined using the T0.5 and T0.75 (in Kelvin) from first and second derivative curves using equation1, where C = -0.007, A = -0.0044, A = -0.0032 kcal/mol/K for heteroduplex, homoduplex, and monomolecular self-structured DNA and RNA melting curves.3

∆H°=C1T0.5-1T0.75                                                                                (1)

Thermodynamic models for duplex formation. Thermo-dynamic parameters for helix formation are obtained using the van't Hoff relationship (equation 2), which relates the equilibrium constant to temperature:

lnK=∆S°R- ∆H°RT                                                                                  (2)

Where ΔS° is the entropy of helix formation, ΔH° is the enthalpy of helix formation, R is the gas constant in kcal/mol, T is the temperature in Kelvin, and K is the equilibrium constant given by equation 3, equation 4, and equation 5 for heteroduplexes, homoduplexes, and monomolecular self-structured RNA respectively.

K=[AB]A[B]                                                                                     (3)

K=[AA]A2                                                                                      (4)

K=[F][U]                                                                                       (5)

For Equation 3 and 4, [A] and [B] are the concentration of different strands, [AB] is the concentration of strand A in a duplex with strand B, and [AA] is the concentration of a self-complementary strand A in a duplex with another self-complementary strand A. For Equation 5, [F] is the concentration of a monomolecular self-structured RNA in the folded state and [U] is the concentration of a monomolecular self-structured RNA in the unfolded state.

MeltR uses three methods based on the van't Hoff equation to calculate thermodynamic parameters: 
(1) fitting melting curves individually, (2) fitting the Tm as a function of temperature, and (3) Global fitting melting curves.

Method 1: fitting melting curves individually. Method 1 fits the absorption as a function of temperature for each sample individually. Base lines are modeled as y= mx + b for the absorbance of the unfolded-single stranded and folded-duplex states (Equation 6).

A= mT+b                                                                                                (6)

The absorption of each sample as a function of temperature is fit to equation 7 using the nls algorithm in base R. SS represents the RNA in the single-stranded state.

A= mDST+bDSfT+ mSST+bSS 1-f(T)                                                                (7)

f(T) is variable, calculated by the analytic solution of the binding constant. MeltR uses Equation 8 for heteroduplexes, equation 9 for homoduplexes, and equation 10 monomolecular self-structured RNA.

fT=2KTCt+ 2-2KTCt + 22-42                                                              (8)

fT=12KTCt+ 2-12KTCt + 22-42                                                              (9)

fT=K(T)KT+1                                                                                      (10 )

Where Ct is the total strand concentration. K(T) is the equilibrium constant as a function of temperature, given by equation 11, where B is 4/Ct, 1/Ct, and 1 for heteroduplexes, homoduplexes, and monomolecular self-structured RNA, respectively.

KT=e∆H°R1Tm-1T+lnB                                                                               (11)

Note, K(T) is in terms of Tm and Ct, instead of the ΔS°, to increase the ease of estimating initial parameters for non-linear regression and to increase the robustness of the nls algorithm. Briefly, the ΔS° is replaced with Tm and Ct by solving equation 2 for the ΔS° at the Tm, where f(T) = 0.5.

The ΔS° of helix formation is calculated from the ΔH° and the Tm.

∆S°= ∆H°Tm+R*lnB                                                                                  (12)

The free energy at 37 °C (ΔG°37) is calculated from the ΔH° and ΔS° of helix formation

∆G°37= ∆H°-310.15*∆S°                                                                             (13)

Error in the ΔS° and ΔG°37 is calculated by propagating variance and covariance in the fit terms ΔH° and Tm, from the non-linear regression model.

σ∆G°37 = ∆S°σ∆H°∆H° 2+σTmTm2-2σ∆H°,Tm∆H°*Tm                                                            (14)

σ∆G°37 = ∆S°σ∆H°∆H° 2+σTmTm2-2σ∆H°,Tm∆H°*Tm                                                             (15)

Method 2: fitting the Tm as a function of Ct. Method 2 uses nls to fit the relationship between 1/Tm and the total strand concentration Ct to equation 15 and 16 for heteroduplexes and homoduplexes, respectively. The Tm of monomolecular, self-structured RNA is independent of Ct so method 2 cannot be used.

1Tm= R∆H°lnCt+∆S°∆H°+ R*ln4                                                                    (16)

1Tm= R∆H°lnCt+∆S°∆H°                                                                                 (17)

The Tm for each sample calculated from method 1 is used for method 2 by default. However, MeltR can use two other methods. The first uses the linear baseline estimates from method 1 to calculate the f(T) using equation 15.

fT= A- mSST+bSS   mDST+bDS - mSST+bSS                                                                  (18)

f(T) is approximately linear in the range of 0.4 to 0.6. Thus, MeltR fits f(T in {0.4 to 0.6}) to y = mT + b using the lm function in base R, which is solved using y = 0.5 to accurately determine the melting temperature for each Ct. Alternatively, method 2 can use the Tm estimated using polynomial regression followed by the first derivative analysis in step 1.

The ΔG°37 is calculated from the ΔH° and the ΔS° using equation 13. Error in the ΔG°37 is calculated by propagating variance and covariance in the fit terms ΔH° and ΔS°.

σ∆G°37 = ∆S°σ∆H°∆H° 2+310.15σ∆S°∆S° 2-2*310.15σ∆H°,∆S°∆H°*∆S°                                             (19)

Method 3: Global fitting. Method 3 fits all curves simultaneously in a global fit using the nls algorithm in base R. In this global fit, equation 2 is plugged directly into equations 8-10, so that the fit is in terms of the ΔS° instead of the Tm, allowing linking of individual curves. The baselines are allowed to vary but ΔH° and ΔS° are constrained to a single value for all curves. For global fitting, the slopes and intercepts of the fits from method 1 are used as initial parameter estimates for the slopes and intercepts of the global fit, and the average of the ΔH° and ΔS° from method 1 are used as initial parameter estimates. The ΔG°37 and error in the ΔG°37 is calculated using the same equations as method 2.

## Data processing by the BLtrimmer

The BLTrimmer function explores the dependence of fit parameters on baseline trimming, starting from an existing meltR.A fit object. The BLTrimmer uses three steps: Rapid testing of baseline combinations, assessment, and a slower treatment of an ensemble of baselines that produce internally consistent thermodynamic energies.

Baseline testing. For the first step, the BLtrimmer generates and then fits a large number of baseline combinations. The user can generate baseline combinations using a fixed or a floating baseline trim. The fixed method uses the same baseline length for each sample to test how baseline length effects the subsequent thermodynamic parameters. The floating method allows different samples to have different baseline lengths. The floating method generates a large number of possible baseline combinations. Thus, the user can only test a small subset of these baseline combinations using a reasonable amount of computational time. Thus, a randomly selected subset of baseline combinations are tested using a fast, lightweight fast. The BLtrimmer fits each sample using method 1, using the global fit from the meltR.A object as initial guesses for the non-linear regression step. Using these good initial guesses reduces the time the nls algorithm needs to find a solution but could bias the result. Next, the BLtrimmer fits each sample using method 2 as described above. ΔH° values from each method and baseline combination are then passed to the assessment step.

Baseline assessment. The user can choose 1 of 3 assessment methods to identify the baseline combinations that produce the most internally consistent thermodynamic parameters. Assessment method 1 tests how well the ΔH° values from each individual sample in method 1 agree with each other. The normalized standard deviation (σ∆H°1) is calculated for each baseline combination, where  σ∆H°1 and μ∆H°1 are the standard deviation and mean ΔH° from the samples in method 1.

σ∆H°1 = σ∆H°1μ∆H°1                                                                                     (20)

σ∆H°1 is then ranked into quantiles on a scale from 0 to 1. For example, if a baseline combination has the 100th smallest σ∆H°1 in a set of 1000 baseline combinations, its quantile (Q∆H°1) is 0.1. Then, baseline combinations with Q∆H°1  values smaller than a user specified value are passed to the ensemble analysis.

Assessment method 2 tests how well the ΔH° values from method 1 and 2 to agree using equation 21, where is the ΔH°2 produced by method 2.

σ∆H°1,2 =2 μ∆H°1-  ∆H°2μ∆H°1+  ∆H°2                                                                        (21)

σ∆H°1,2 is then ranked into quantiles on a scale from 0 to 1 to produce Q∆H°1,2 values and baseline combinations with Q∆H°1,2  values smaller than a user specified value are passed to the ensemble analysis.

Assessment method 3 combines assessment methods 1 and 2 using a statistic called the error distance (equation 22).

σD =Q∆H°12 + Q∆H°1,22                                                                            (22)

σD is then ranked into quantiles on a scale from 0 to 1 to produce QD values and baseline combinations with QD  values smaller than a user specified value are passed to the ensemble analysis.

Ensemble analysis. Baselines that pass the assessment criterion are treated as an ensemble of equally feasible baseline combinations. Each baseline combination is passed to meltR.A and fit, which takes considerably longer than the lightweight analysis in the testing step because initial guesses are predicted for each trim using the protocol described for meltR.A. The average of resulting thermodynamic parameters from each baseline are reported with 95% confidence intervals.

## Data Modeling

Data were modeled to produce simulated melting curves with realistic baselines and random scatter for each two-state folding RNA sequence in two published sources.4,5 First, a series of 9 samples were generated with Cts that evenly span Ct = 0.2/ε and 20/ε on a ln transformed scale, where ε is the molar extinction coefficient of the RNA, used as the extinction coefficient of the upper baseline. Samples were placed into virtual 1, 0.5, and 0.1 cm pathlength (l) cuvettes so that the calculated absorbance was between 0.2 and 2. The extinction coefficient of the lower baseline was generated by multiplying ε by 0.6. Random baseline slopes (m) were seeded into the modeled data using a Gaussian distribution with the rnorm function in base R, with a mean of 0.00111 and a standard deviation of 0.00078. This mean and standard deviation was calculated from an ensemble of 9 fits to real data. The intercept (b) was generated for each baseline using equation 23.

b =Ct*l*ε-90*m                                                                            (23)

Then, parameters were plugged into equation 7, using the ΔH° and ΔS° reported in the aforementioned papers to generate melting curves that span 5 to 95 °C with 0.5 °C increments. Random absorbance scatter was seeded into the melting curves using the rnorm function with a mean of 0 and a standard deviation of 0.0005.

# Supplementary information figures

Figure S1  Precanned figures and tables for assessing the quality of the results are written as PDF and comma separated value files by meltR.A, respectively. Black circles represent data points and red lines represent fits. (A) Raw data fit using method 1. (B) Normalized data fit using method 1. Data were normalized to calculate the absorptivity at each temperature by dividing the raw absorbance by Ct and the pathlength of the cuvette. (C) Table summarizing the individual fits for method 1. Sample is the sample identifier in the data. Ct is the total RNA strand concentration, Ct. H is the ΔH°, in kcal/mol. S is the ΔS° in cal/mol/K. G is the ΔG°37, in kcal/mol. Tm is the Tm of the curve in °C. (D) Plot of 1/Tm versus the natural log of the Ct , used to determine folding energies for method 2. (E) Raw data fit using method 3, global fitting. (F) Normalized data fit using method 3. Data were normalized as described in A. (G) Table summarizing results from the three methods labeled as in A. SE stands for the standard error, which is calculated from the regression models and represent the precision of the parameter given the data.

Figure S2  One-thousand randomly generated baseline combinations provides accurate thermodynamic parameters and is computationally achievable. (A) System time for the BLTrimmer as a function of the number of baseline combinations tested in each run. (B-E) Convergence of thermodynamic parameters as a function of the number of baseline combinations tested in a BLTrimmer run. Points represent the mean value and error bars represent 95% confidence intervals.

Figure S3  Precanned BLtrimmer outputs for assessing the quality of the results are written as a PDF file. Blue lines and points represent the 25% of baseline combinations that produce parameters that exhibit the best internal consistency across the data set. Each point comes from fitting a combination of baselines with method 1 or 2. (A) Distribution of the standard deviation of ΔH° values, determined by fitting 100 baseline combinations with method 1. Low standard deviation indicates that the baseline combination exhibits high agreement ΔH° calculated using method 1. (B) Distribution of ΔH° agreement between method 1 and method 2 determined by fitting 100 baseline combination. Low difference indicates that a baseline combination exhibits high agreement in ΔH° values calculated using methods 1 and 2. (C) Ranked standard deviation of ΔH° values in the method 1 fit versus the ranked difference between ΔH° values from method 1 and 2, for each 100 baseline combinations. Error distance, which is used to identify the 25% of baseline combinations that produce parameters that exhibit the best internal consistency, is the Pythagorean distance between a point and the origin of the plot. (D) The average ΔH° produced by method 1 versus the standard deviation of ΔH° values produced by method 1 for each baseline combination. (E) The average ΔH° produced by method 1 (Black) or the ΔH° produced by method 2, versus the ΔH° agreement between methods 1 and 2, for 100 baseline combinations. (F) The standard deviation of ΔH° values produced by method 1 versus ΔH° agreement between methods 1 and 2.

Figure S4  MeltR accurately fits modeled data with no user intervention. (A) ΔH° determined using meltR.A followed by the BLtrimmer to fit modeled data, versus the known ΔH°. Vertical error bars represent 95% confidence intervals. (B) ΔS° determined using meltR.A followed by the BLtrimmer to fit modeled data, versus the known ΔH°. Error bars are the same as A. (C) ΔG°37 determined using meltR.A followed by the BLtrimmer to fit modeled data, versus the known ΔH°. Error bars are the same as A. (D) Tm at 0.1 mM determined using meltR.A followed by the BLtrimmer to fit modeled data, versus the known ΔH°. Error bars are the same as A. (E) Agreement between MeltWin and MeltR methods and the consensus nearest neighbor model.4,6

Table S1  Results for fitting raw absorbance melting curves with MeltR and MeltWin.

| Helix | MeltR ΔH° Method 1a (kcal/mol) | MeltR ΔH° Method 2b (kcal/mol) | MeltR ΔH° Method 3c (kcal/mol) |  |  | MeltR ΔS° Method 1a (cal/mol/K) | MeltR ΔS° Method 2b (cal/mol/K) | MeltR ΔS° Method 3c (cal/mol/K) |  |  | MeltR ΔG°37 Method 1a (kcal/mol) | MeltR ΔG°37 Method 2b (kcal/mol) | MeltR ΔG°37 Method 3c (kcal/mol) |  |  | MeltR Tmd Method 1a (°C) | MeltR Tmd Method 2b (°C) | MeltR Tmd Method 3c (°C) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5’-CGCGCG-3’e ‘3’-GCGCGC-3 ’ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5’-ACCGGU-3’e 3’-UGGCCA-3’ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5’-CCAUGG-3’e 3’-GGUACC-5’ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5’-GAUAUAUC-3’e 3’-CUAUAUAG-5’ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5’-GCAAUUGC-3’e 3’-CGUUAACG-5’ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5’-CGAAAGGU-3’f 3’-GCUUUCCA-5’ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5’-CUGAGUC-3’f 3’-GACUCAG-5’ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5’-CGUUGC-3’f 3’-GCAACG-5’ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| F-CGAAAGGU-3’f Q-GCUUUCCA-5’ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| F-CUGAGUC-3’ f Q-GACUCAG-5’ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| F-CGUUGC-3’f Q-GCAACG-5’ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

aDetermined with method 1, fitting curves individually and averaging the results.

bDetermined with method 2, 1/Tm versus lnCt analysis.

cDetermined with method 3, globally fitting all curves with the same non-linear model.

dExpectation maximized Tm at a Ct of 0.1 mM.

eData compiled from a published source.1

fData compiled in this work.

# Supplementary information references

(1)	Handbook of Biochemistry and Molecular Biology : Nucleic Acids; Cleveland : CRC Press, 1975.

(2)	Puglisi, J. D.; Tinoco, I. Absorbance Melting Curves of RNA. In Methods in Enzymology; RNA Processing Part A: General Methods; Academic Press, 1989; Vol. 180, pp 304–325. https://doi.org/10.1016/0076-6879(89)80108-9.

(3)	Gralla, J.; Crothers, D. M. Free Energy of Imperfect Nucleic Acid Helices: III. Small Internal Loops Resulting from Mismatches. J. Mol. Biol. 1973, 78 (2), 301–319. https://doi.org/10.1016/0022-2836(73)90118-6.

(4)	Xia, T.; SantaLucia, J.; Burkard, M. E.; Kierzek, R.; Schroeder, S. J.; Jiao, X.; Cox, C.; Turner, D. H. Thermodynamic Parameters for an Expanded Nearest-Neighbor Model for Formation of RNA Duplexes with Watson−Crick Base Pairs †. Biochemistry 1998, 37 (42), 14719–14735. https://doi.org/10.1021/bi9809425.

(5)	Antao, V. P.; Tinoco, I., Jr. Thermodynamic Parameters for Loop Formation in RNA and DNA Hairpin Tetraloops. Nucleic Acids Res. 1992, 20 (4), 819–824. https://doi.org/10.1093/nar/20.4.819.

(6)	Adams, M. S.; Znosko, B. M. Thermodynamic Characterization and Nearest Neighbor Parameters for RNA Duplexes under Molecular Crowding Conditions. Nucleic Acids Res. 2019, 47 (7), 3658–3666. https://doi.org/10.1093/nar/gkz019.